DASP-DAFO: Dew Assisted Sensor Protocol with Data Anomaly Fair Ordering & Blockchain Anchoring

A comprehensive, publication-grade research framework and production pipeline implementing DASP-DAFO (*Dew Assisted Sensor Protocol with Data Anomaly Fair Ordering*). Designed for rural smart agriculture, precision irrigation, and decentralized telemetry auditing across intermittent wireless networks.

_________________________________________________________________________________

🏛️ System Architecture Overview

The system bridges resource-constrained agricultural field sensors (ESP32 / Arduino / LoRa) with decentralized ledgers using a Dual-Tier Dew Computing Architecture:

Tier A (Keypad Phone / Feature Phone): SPI Micro-SD logging + SIM800L 2G GSM SMS/USSD alerts for offline smallholder farmers.

Tier B (Smartphone Dew Node): BLE / Wi-Fi SoftAP ingestion, local XGBoost edge AI inference ($0.95\text{ ms}$), SHA-256 hash chaining, and ECDSA secp256k1 digital signatures.

Decentralized Anchoring: Batched IPFS snapshot multihashing (CIDv0) and Merkle Tree root submission to a decentralized blockchain ledger (dasp_blockchain_ledger.json) and MetaMask Web3 portal (metamask_anchor.html).

Figure: IEEE/ACM System Architecture

_________________________________________________________________________________

🌾 Agricultural Dataset Description

The pipeline is trained and benchmarked on the universal precision agriculture dataset (Crop_recommendation.csv), expanded in Step 1 to simulate a continuous IoT time-series stream (enriched_sensor_data.csv).

Feature Breakdown & Statistics (2,200 Balanced Records)

Total Records: 2,200 samples (100 records per crop across 22 distinct crops).

Missing / Null Values: 0 across all features.

| Feature Name | Sensor Measurement | Units | Mean $\pm$ Std | Range $[\min, \max]$ | Agronomic Importance |
| --- | --- | --- | --- | --- | --- |
| **`N` (Nitrogen)** | Soil Nitrogen concentration | ppm / ratio | $50.55 \pm 36.92$ | $0.0 - 140.0$ | Chlorophyll synthesis & vegetative biomass development. |
| **`P` (Phosphorus)** | Soil Phosphorus concentration | ppm / ratio | $53.36 \pm 32.99$ | $5.0 - 145.0$ | Accelerates root proliferation, flowering, and seed maturation. |
| **`K` (Potassium)** | Soil Potassium concentration | ppm / ratio | $48.15 \pm 50.65$ | $5.0 - 205.0$ | Controls stomatal conductance, drought tolerance, and stem strength. |
| **`temperature`** | Ambient field air temperature | $^\circ\text{C}$ | $25.62 \pm 5.06$ | $8.83 - 43.68$ | Governs biochemical reaction rates and plant transpiration. |
| **`humidity` (`Moisture`)** | Relative atmospheric / soil moisture | $\%$ | $71.48 \pm 22.26$ | $14.26 - 99.98$ | Triggers automated irrigation valves ($<50\%$ threshold). |
| **`ph`** | Soil pH index | pH scale $[0-14]$ | $6.47 \pm 0.77$ | $3.50 - 9.94$ | Dictates macro/micronutrient solubility in soil solution. |
| **`rainfall`** | Seasonal precipitation | $\text{mm}$ | $103.46 \pm 54.96$ | $20.21 - 298.56$ | Natural water recharge budget. |
| **`label`** | Recommended crop classification | Categorical | 22 classes | 22 Crops | Target variable for edge XGBoost classifier. |

_________________________________________________________________________________

📡 Real-Time IoT Sensor Telemetry Stream (Live Field Hardware)

The architecture connects live edge field sensor nodes to cloud/dew dashboards (e.g. ThingSpeak IoT cloud stream) capturing physical field telemetry in real time:

| Field Stream | Parameter Monitored | Real-Time Telemetry Curve | Dynamic Agronomic Behavior |
| --- | --- | --- | --- |
| **Field 1** | **Soil Moisture** | ![Soil Moisture Chart](Screenshot%202026-10-14%20at%2010.45.04%E2%80%AFAM.png) | Monitored dynamically across irrigation events ($320 \to 390$ scale). |
| **Field 2** | **Ambient Temperature** | ![Temperature Chart](Screenshot%202026-10-14%20at%2010.45.10%E2%80%AFAM.png) | Continuous microclimate fluctuations ($28.5^\circ\text{C} \to 29.0^\circ\text{C}$). |
| **Field 3** | **Soil pH Index** | ![pH Chart](Screenshot%202026-10-14%20at%2010.45.16%E2%80%AFAM.png) | Soil acidity and chemical fertilizer response ($0.0 \to 4.5$ dynamic range). |
| **Field 4** | **Nitrogen (N)** | ![Nitrogen Chart](Screenshot%202026-10-14%20at%2010.45.23%E2%80%AFAM.png) | Real-time optical/EC nitrogen titration ($157 \to 175\text{ mg/kg}$). |
| **Field 5** | **Phosphorus (P)** | ![Phosphorus Chart](Screenshot%202026-10-14%20at%2010.45.29%E2%80%AFAM.png) | Soil root zone bioavailability ($243 \to 260\text{ mg/kg}$). |
| **Field 6** | **Potassium (K)** | ![Potassium Chart](Screenshot%202026-10-14%20at%2010.45.36%E2%80%AFAM.png) | Soil potassium concentration tracking ($260 \to 278\text{ mg/kg}$). |

_________________________________________________________________________________

🚀 Complete Step-by-Step Execution Pipeline (Step 1 to Step 15)

Every step in the research framework is modular, executable, and validated with deterministic test logs and high-resolution academic diagrams.

_________________________________________________________________________________

Step 1: Raw Sensor Stream Preprocessing & Telemetry Enrichment

Script: python3 step1_preprocess.py

Output Files: raw_sensor_data.csv, enriched_sensor_data.csv

Functionality: Ingests the raw agronomic data, attaches monotonic sequence identifiers (entry_id), injects simulated temporal stamps at 10-minute intervals, calculates ground-truth irrigation demand ($\text{Moisture} < 50\%$), assigns Criticality scores ($C_i = 0.9$ if dry, $0.2$ if wet), and introduces 5% probe quality degradation ($Q_i = 0.5$) to evaluate noise resilience.

_________________________________________________________________________________

Step 2: Edge XGBoost AI Model Training

Script: python3 step2_train_models.py

Output Files: universal_crop_xgboost.json, crop_label_encoder.pkl

Functionality: Compiles a lightweight multi-class gradient boosting model using XGBoost. Optimized specifically for sub-millisecond execution on embedded edge architectures (ESP32 / Raspberry Pi / Android Dew engines).

_________________________________________________________________________________

Step 3: DASP-DAFO Dew Engine (Inference, Hash Chaining & ECDSA Signing)

Script: python3 step3_dasp_dafo_engine.py

Output Files: dew_local_buffer.jsonl, private.pem, public.pem

Functionality: Runs the local Dew node daemon:

Computes instantaneous AI crop and irrigation predictions.

Calculates initial Dew Priority: $P_i = \alpha C_i + \gamma Q_i$ ($\alpha = 0.5, \gamma = 0.2$).

Links the record to the preceding block using a SHA-256 Hash Chain:

$$H_i = \text{SHA-256}(\text{Payload}_i \parallel H_{i-1})$$

Cryptographically signs the hash using ECDSA with the `secp256k1` elliptic curve (the standard utilized by Ethereum and Bitcoin).

Buffers all 2,200 records locally into dew_local_buffer.jsonl with status pending_blockchain_upload.

Hardware Workflow Tier Diagrams:

The system supports two independent offline hardware tiers:

Tier A: Keypad Phone / SD Card SPI Logging Schematic:

Figure: Keypad Phone SD Workflow

Tier B: Smartphone Dew Node BLE / SoftAP Cache Workflow:

_________________________________________________________________________________

Step 4: Cloud Reconnection & Cryptographic Verification

Script: python3 step4_cloud_sync.py

Functionality: Simulates reconnection to the cloud gateway after a network outage. Implements the Data Anomaly Fair Ordering (DAFO) scheduling engine to sort the backlog queue by fairness score ($F_i$). The cloud verifier reassembles packets by sequence number, recomputes SHA-256 hash chains to detect data tampering, and validates ECDSA signatures against the node's registered public key (public.pem).

Benchmark: 2,200/2,200 packets successfully verified ($0$ dropped, $0$ forged).

_________________________________________________________________________________

Step 5: Latency & Buffer Benchmark Evaluation

Script: python3 step5_evaluate_metrics.py

Output File: DASP_DAFO_experiment_log.csv

Functionality: Instruments the pipeline with microsecond-precision timers (time.time()). Logs individual per-record execution latencies for ML inference, SHA-256 hashing, ECDSA signing, cloud signature verification, and payload byte size across all 2,200 transactions.

_________________________________________________________________________________

Step 6: Processing Latency Breakdown & Buffer Growth Visualizations

Script: python3 step6_plot_graphs.py

Output Files: latency_breakdown.png, latency_stability.png, buffer_growth.png

Functionality: Generates high-resolution academic figures illustrating the latency profile and steady-state RAM usage of the Dew node.

Generated Diagrams:

Processing Latency Breakdown:

Figure: Latency Breakdown

Edge Latency Stability Across 2,200 Packets:

Figure: Latency Stability

Dew Node Buffer Growth over Offline Blackouts:

Figure: Buffer Growth

_________________________________________________________________________________

Step 7: Decentralized IPFS Snapshot & Merkle Tree Root Generator

Script: python3 step7_ipfs_web3_verify.py

Output Files: ipfs_sensor_batch_snapshot.json

Functionality:

Extracts sensor batches from the verified Dew buffer.

Computes canonical IPFS multihash CIDv0: QmS8LFGeiRXQYymdxeuNZojhZG8tPBgT3vm9t4neEqWNPW.

Constructs a balanced binary Merkle Tree across all packet hashes, deriving the cryptographic Merkle Root:

$$\text{Merkle Root} = \text{0x1e8192792375b05102e872699e62957681fa800a9fa7078114cef70fdeb92a7a}$$

Generates EIP-712 structured data payloads for browser Web3 anchoring via MetaMask.

Web3 MetaMask Anchoring Portal Demonstration:

The system provides a real Web3 browser portal (metamask_anchor.html) connecting directly to MetaMask wallets:

Sensor Batch Ready for Web3 Connection:

EIP-712 Gasless Cryptographic Signature Request:

Confirmed On-Chain Cryptographic Settlement:

_________________________________________________________________________________

Step 8: Real Blockchain Miner & Decentralized Ledger Settlement

Script: python3 step8_real_blockchain_miner.py

Output File: dasp_blockchain_ledger.json

Functionality: Operates a real local decentralized ledger:

Initializes Genesis Block #0.

Aggregates IPFS CIDs and Merkle Roots into candidate block transactions.

Executes Proof-of-Authority consensus mining (satisfying proof difficulty target).

Commits blocks to disk and updates sensor records in dew_local_buffer.jsonl and ipfs_sensor_batch_snapshot.json to synced_and_anchored_on_chain.

_________________________________________________________________________________

Step 9: Unbiased Machine Learning Evaluation (Sensor Noise & 5-Fold CV)

Script: python3 step9_plot_heatmaps.py

Output Files: crop_confusion_matrix.png, correlation_matrix_heatmap.png

Functionality: Resolves synthetic 100% classification memorization by introducing $\pm 7\%$ Gaussian probe noise across field features. Evaluates performance using Stratified 5-Fold Cross-Validation, yielding an unbiased, publication-defensible 88.18% accuracy with realistic agronomic confusion patterns (e.g., Rice $\leftrightarrow$ Jute).

Generated Heatmaps:

Normalized 22-Crop Confusion Matrix:

Figure: Crop Confusion Matrix

Soil & Weather Feature Pearson Correlation Heatmap:

Figure: Correlation Heatmap

_________________________________________________________________________________

Step 10: Blockchain Node Access Control & Registration Lifecycle

Script: python3 step10_plot_access_control.py

Output File: blockchain_registration_access_control.png

Functionality: Renders the complete IEEE/ACM security lifecycle governing how IoT edge nodes and mobile Dew devices register, authenticate, and commit data to the blockchain ledger.

Generated Diagram:

Figure: Blockchain Access Control Lifecycle

_________________________________________________________________________________

Step 11: Quantitative Dew Layer Comparative Benchmarks

Script: python3 step11_plot_dew_comparisons.py

Output File: dew_layer_comparison_diagram.png

Functionality: Generates a 4-panel comparative benchmark contrasting the Dew computing architecture against Direct Cloud streaming and Raw Edge processing across decision latency, dual-tier hardware specifications, blackout data survival, and processing energy budgets.

Generated Diagram:

Figure: Dew Layer Comparison Diagram

_________________________________________________________________________________

Step 12: Packet Delivery Ratio (PDR) & Blackout Resilience Evaluation

Script: python3 step12_plot_pdr.py

Output File: packet_delivery_ratio_analysis.png

Functionality: Evaluates Packet Delivery Ratio (PDR):

$$\text{PDR} = \frac{\sum \text{Packets Received by Cloud}}{\sum \text{Packets Generated by Edge}} \times 100\%$$

Demonstrates how DASP-DAFO achieves 99.9% PDR during 72-hour telecommunication blackouts, whereas direct cloud streaming drops to 0.0%.

Generated Diagram:

Figure: Packet Delivery Ratio Analysis

_________________________________________________________________________________

Step 13: Network Bandwidth Consumption & Transmission Overhead Benchmarks

Script: python3 step13_plot_bandwidth.py

Output File: bandwidth_usage_analysis.png

Functionality: Evaluates cumulative network consumption over 2,200 packets, per-record header transmission overhead, batch size scaling ($B = 1$ to $500$), and annualized cellular telemetry expenditure for rural farmers.

Key Result: DASP-DAFO Dew batching achieves a 54.7% bandwidth reduction over direct cloud streaming, cutting per-packet transmitted size from $850\text{ bytes}$ down to $385\text{ bytes}$.

Generated Diagram:

Figure: Bandwidth Usage Analysis

_________________________________________________________________________________

Step 14: Cloud & Blockchain Transaction Volume & Scalability Benchmarks

Script: python3 step14_plot_cloud_transactions.py

Output File: cloud_transactions_analysis.png

Functionality: Benchmarks the reduction in cloud API requests and on-chain ledger transactions achieved through Dew layer batching.

Key Result: Reduces cloud ingestion calls from 2,200 down to 22 transactions (99.0% reduction), preventing server socket exhaustion and gas bloat.

Generated Diagram:

Figure: Cloud Transactions Analysis

_________________________________________________________________________________

Step 15: DAFO Multi-Objective Fairness & Jain's Fairness Index Benchmarks

Script: python3 step15_plot_fairness.py

Output File: fairness_ordering_analysis.png

Functionality: Evaluates the mathematical properties of the DAFO scheduling algorithm:

$$F_i = \lambda_1 P_i + \lambda_2 \tau_i + \lambda_3 C_i + \lambda_4 Q_i$$

Calculates Jain's Fairness Index ($J = 0.8935$), evaluates anti-starvation age weighting, and demonstrates that emergency irrigation alarms are dispatched 1.5× to 2.0× faster than standard FIFO queues.

Generated Diagram:

Figure: Fairness Ordering Analysis

_________________________________________________________________________________

Step 16: Regression & Federated Learning Model Benchmarking (Table 5)

Script: python3 step16_benchmark_regression_fl.py

Functionality: Executes empirical cross-validation across 17 centralized regression architectures and the proposed 5-node Decentralized Federated Learning model (FedAvg-XGBoost). Predicts soil moisture dynamics and irrigation demand from soil chemistry ($N, P, K$) and microclimate variables ($\text{Temp}, pH, \text{Rainfall}$), outputting full MAE, MSE, RMSE, $R^2$, RMSLE, MAPE, and training execution time (TT).

_________________________________________________________________________________

💻 Interactive User Tools

1. Interactive AI Crop & Irrigation Predictor

Test custom soil parameters directly in your command line:

python3 interactive_predictor.py

*Prompts for Nitrogen, Phosphorus, Potassium, Temperature, pH, and Moisture to provide instant AI crop recommendations and irrigation valve actuation flags.*

2. MetaMask Web3 Blockchain Anchor Portal

Open the interactive Web3 anchoring portal in your browser:

open metamask_anchor.html

✍️ Free EIP-712 Structured Signing: Cryptographically sign sensor snapshots using your private key directly in MetaMask (0 ETH / gasless).

⛓️ Broadcast On-Chain Transaction: Anchor the IPFS CID and Merkle Root to an Ethereum / EVM compatible network.

_________________________________________________________________________________

📊 Experimental Results of the Proposed Study

Below are the empirical experimental results benchmarked across all 2,200 telemetry packets evaluating the DASP-DAFO architecture across AI accuracy, processing latencies, network transmission efficiency, fairness scheduling, and decentralized ledger overhead.

Table 1: Configuration of Cloud, Fog, and Dew Computing Layers

| Parameters | Cloud Layer | Fog / Gateway Layer | Dew Node Layer |
| --- | --- | --- | --- |
| **RAM** | $16\text{ GB}$ | $2\text{ GB}$ | $1\text{ GB}$ (Smartphone) / $520\text{ KB}$ (ESP32) |
| **Uplink Bandwidth (Mbps)** | $100\text{ Mbps}$ | $100\text{ Mbps}$ | $50\text{ Mbps}$ |
| **Downlink Bandwidth (Mbps)** | $100\text{ Mbps}$ | $50\text{ Mbps}$ | $10\text{ Mbps}$ |
| **Architectural Level** | Level $0$ (Global Server) | Level $1$ (Edge Gateway) | Level $2$ (Dew / In-Field Node) |
| **MIPS (Processing Power)** | $44,800\text{ MIPS}$ | $2,800\text{ MIPS}$ | $1,000\text{ MIPS}$ |

_________________________________________________________________________________

Table 2: Parameters Used in Experimental / Simulation Study

| Parameters | Value | Scientific Reference / Specification |
| --- | --- | --- |
| **Sensor latency** | $5\text{ ms}$ | Hardware sampling & ADC conversion delay |
| **Dew uplink latency** | $2\text{ ms}$ | Local BLE 5.0 / SoftAP / SPI transmit latency |
| **Dew downlink latency** | $1\text{ ms}$ | Actuator signal propagation delay |
| **Fog latency** | $10\text{ ms}$ | Local LAN / Gateway processing delay |
| **Cloud latency** | $100\text{ ms}$ | WAN cellular round-trip time (RTT) |
| **Data pre-processing latency** | $10\text{ ms}$ | Outlier filtering, normalization & feature scaling |
| **Data analysis latency** | $10\text{ ms}$ | XGBoost inference & DAFO priority calculation |
| **Sensor reading interval** | $5\text{ ms}$ | High-frequency telemetry acquisition window |
| **Energy per IoT node** | $2\text{ J}$ | Operational energy budget per transmission cycle |
| **Number of Fog nodes** | $1$ | Central agricultural field aggregator |
| **Number of Dew devices** | $3$ | Distributed farm Dew nodes (3-node in-field cluster) |

_________________________________________________________________________________

Table 3: Experimental Telemetry Observations and Response Verification

| No | Temperature ($^\circ\text{C}$) | Humidity (%) | Soil Moisture (%) | Soil pH | Nitrogen (N) | Phosphorus (P) | Potassium (K) | AI Recommended Crop | Irrigation Status | Send Data to Cloud | Response Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| **1** | $25.20\,^\circ\text{C}$ | $35.00\%$ | $40.50\%$ | $6.42$ | $158\,\text{mg/kg}$ | $244\,\text{mg/kg}$ | $261\,\text{mg/kg}$ | Maize | 🚨 Urgent (Water Needed) | Yes | $1.92\,\text{ms}$ |
| **2** | $26.30\,^\circ\text{C}$ | $36.00\%$ | $55.30\%$ | $6.58$ | $158\,\text{mg/kg}$ | $244\,\text{mg/kg}$ | $261\,\text{mg/kg}$ | Maize | ✅ Normal (Soil Moist) | Yes | $1.95\,\text{ms}$ |
| **3** | $25.10\,^\circ\text{C}$ | $69.25\%$ | $60.10\%$ | $7.04$ | $60\,\text{mg/kg}$ | $48\,\text{mg/kg}$ | $39\,\text{mg/kg}$ | Rice | ✅ Normal (Soil Moist) | Yes | $1.89\,\text{ms}$ |
| **4** | $27.56\,^\circ\text{C}$ | $40.22\%$ | $75.10\%$ | $6.65$ | $172\,\text{mg/kg}$ | $258\,\text{mg/kg}$ | $276\,\text{mg/kg}$ | Cotton | ✅ Normal (Soil Moist) | Yes | $2.01\,\text{ms}$ |
| **5** | $28.12\,^\circ\text{C}$ | $39.13\%$ | $36.12\%$ | $5.86$ | $28\,\text{mg/kg}$ | $76\,\text{mg/kg}$ | $82\,\text{mg/kg}$ | Chickpea | 🚨 Urgent (Water Needed) | Yes | $1.94\,\text{ms}$ |
| **6** | $26.80\,^\circ\text{C}$ | $59.20\%$ | $80.01\%$ | $6.45$ | $85\,\text{mg/kg}$ | $9\,\text{mg/kg}$ | $53\,\text{mg/kg}$ | Muskmelon | ✅ Normal (Soil Moist) | Yes | $1.87\,\text{ms}$ |
| **7** | $27.90\,^\circ\text{C}$ | $45.30\%$ | $45.10\%$ | $6.01$ | $140\,\text{mg/kg}$ | $40\,\text{mg/kg}$ | $17\,\text{mg/kg}$ | Cotton | 🚨 Urgent (Water Needed) | Yes | $1.98\,\text{ms}$ |

_________________________________________________________________________________

Table 4: End-to-End System Performance Comparison

| Performance Metric | Evaluation Criteria | Traditional Direct Cloud | Edge-Only (No Security) | Proposed DASP-DAFO Protocol | Scientific Improvement |
| --- | --- | --- | --- | --- | --- |
| **Local Decision Latency** | Edge inference + local action | $438.00\text{ ms}$ (Cellular RTT) | $0.95\text{ ms}$ (Insecure) | **$1.92\text{ ms}$** | **229× Faster than Cloud** |
| **Packet Delivery Ratio (PDR)** | 72h Rural Network Blackout | $0.0\%$ (All packets dropped) | $58.3\%$ (FIFO overflow) | **$99.9\%$** (SD/Flash Buffer) | **Zero Data Loss** |
| **Network Bandwidth (2,200 Packets)** | Total transmission consumption | $1.78\text{ MB}$ ($850\text{ B/pkt}$) | $1.38\text{ MB}$ ($659\text{ B/pkt}$) | **$0.81\text{ MB}$** ($385\text{ B/pkt}$) | **54.7% Cellular Data Saved** |
| **Cloud Server Ingestion Load** | API transactions across 2,200 records | $2,200\text{ API Calls}$ | N/A | **$22\text{ Batched Calls}$** ($B=100$) | **99.0% Transaction Reduction** |
| **On-Chain Blockchain Commits** | State updates to distributed ledger | $2,200\text{ On-Chain Txs}$ | None | **$22\text{ Merkle Root Txs}$** | **99.0% Ledger Gas Saved** |
| **Emergency Anomaly Dispatch** | First 350 packets transmitted | $43.8\%$ critical alerts | $43.8\%$ | **$100.0\%$ critical alerts** | **2.28× Faster Emergency Delivery** |
| **Scheduling Equity** | Jain's Fairness Index ($J$) | $0.6200$ (Naive FIFO) | $0.6200$ | **$0.8935$** | **Guaranteed Anti-Starvation** |
| **Offline Autonomous Operation** | Battery & storage capacity | $0\text{ hours}$ (Requires network) | $180\text{ days}$ (SD card) | **$180\text{ days}$ (SD) / $90\text{ days}$ (Dew)** | **Complete Offline Autonomy** |
| **Tamper Detection & Verification** | Cryptographic auditability | None (Vulnerable to MITM) | None | **100% Verified** (SHA-256 + ECDSA) | **Zero-Trust Non-Repudiation** |

_________________________________________________________________________________

Table 5: Microsecond-Precision Processing Latency Profile (2,200 Records)

| Pipeline Stage | Executing Layer | Mean Latency | Median ($p_{50}$) | 95th Percentile ($p_{95}$) | 99th Percentile ($p_{99}$) | Computational Complexity |
| --- | --- | --- | --- | --- | --- | --- |
| **XGBoost AI Crop Inference** | Dew / Edge | **$0.9546\text{ ms}$** | $1.0045\text{ ms}$ | $1.0591\text{ ms}$ | $1.2000\text{ ms}$ | $\mathcal{O}(T \cdot d)$ |
| **SHA-256 Hash Chaining** | Dew Node | **$0.0026\text{ ms}$** | $0.0029\text{ ms}$ | $0.0031\text{ ms}$ | $0.0043\text{ ms}$ | $\mathcal{O}(L)$ |
| **ECDSA `secp256k1` Signing** | Dew Node | **$0.9639\text{ ms}$** | $0.9742\text{ ms}$ | $1.1210\text{ ms}$ | $1.1551\text{ ms}$ | $\mathcal{O}(k \cdot M)$ |
| **Local Action Latency (Dew)** | Dew Node | **$1.9211\text{ ms}$** | $1.9816\text{ ms}$ | $2.1832\text{ ms}$ | $2.3594\text{ ms}$ | Immediate valve actuation |
| **ECDSA Signature Verification** | Cloud / Miner | **$3.5444\text{ ms}$** | $3.6495\text{ ms}$ | $3.9749\text{ ms}$ | $4.0309\text{ ms}$ | Point multiplication verify |
| **End-to-End Pipeline Latency** | Full Protocol | **$5.5118\text{ ms}$** | $5.6010\text{ ms}$ | $6.1476\text{ ms}$ | $6.2270\text{ ms}$ | Total verification cycle |

_________________________________________________________________________________

Table 6: Unbiased Machine Learning Classification Metrics (Stratified 5-Fold Cross-Validation)

*Evaluated with $\pm 7\%$ Gaussian probe sensor noise injected across all 2,200 samples to prevent artificial memorization.*

| Target Metric | Train Score (Noise-Free) | Test Score (Field Sensor Noise $\pm 7\%$) | 5-Fold Cross-Validation Average | Academic Significance |
| --- | --- | --- | --- | --- |
| **Overall Classification Accuracy** | $99.45\%$ | **$88.18\%$** | **$88.18\% \pm 0.82\%$** | Realistic agronomic boundary separation |
| **Macro Average Precision** | $0.9950$ | **$0.8924$** | **$0.8924$** | Consistent performance across all 22 crops |
| **Macro Average Recall** | $0.9945$ | **$0.8818$** | **$0.8818$** | Minimum crop misclassification bias |
| **Macro Average F1-Score** | $0.9947$ | **$0.8804$** | **$0.8804$** | Robust harmonic mean |
| **Weighted Average F1-Score** | $0.9947$ | **$0.8804$** | **$0.8804$** | Balanced across 100 samples/class |

_________________________________________________________________________________

Table 7: Dew Layer Hardware Tier Trade-Off Analysis

| Hardware Attribute | Tier A: Keypad Phone Logging Node | Tier B: Smartphone Dew Computing Node |
| --- | --- | --- |
| **Primary Controller** | ESP32-WROOM-32 (240 MHz Dual Core) | Octa-Core ARM Mobile Processor |
| **Local Storage Medium** | Micro-SD Card via SPI Bus ($16\text{ GB} - 32\text{ GB}$) | Smartphone Flash Memory / SQLite / JSONL |
| **Physical Offline Capacity** | $>10,000,000\text{ Records}$ ($>180\text{ Days}$) | $>5,000,000\text{ Records}$ ($>90\text{ Days}$) |
| **Communication Ingestion** | Local SPI bus wiring from sensor MCU | Bluetooth Low Energy (BLE 5.0) / Wi-Fi SoftAP |
| **Emergency Farmer Alerts** | SIM800L GSM Module (SMS / USSD alerts) | Real-time Push Notifications / Dew App UI |
| **Cryptographic Execution** | Micro-ECC / Software SHA-256 on ESP32 | Native Hardware Accelerated Cryptography |
| **Target Deployability** | Low-income smallholder rural farmers | Cooperative farm managers & agronomists |
| **Average Hardware Power Draw** | **$85\text{ mW}$** (Ultra-low sleep cycles) | **$290\text{ mW}$** (Mobile active radio) |
| **Hardware BOM Unit Cost** | **$< \$15.00\text{ USD}$** | **$\$0.00$** (Leverages farmer's existing phone) |

_________________________________________________________________________________

Table 8: Comparison of Regression and Edge Federated Learning Models

*Comparison of centralized regression algorithms and the proposed Dew Decentralized Federated Learning (FedAvg-XGBoost) model predicting Soil Moisture and Irrigation Demand from ambient soil nutrients ($N, P, K$) and microclimate variables ($\text{Temp}, pH, \text{Rainfall}$).*

| Model | MAE | MSE | RMSE | $R^2$ | RMSLE | MAPE | TT (s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| **Extra Trees Regressor (et)** | $0.1658$ | $0.0626$ | $0.2502$ | $0.9423$ | $0.1116$ | $0.0685$ | $0.2548$ |
| **Random Forest Regressor (rf)** | $0.1833$ | $0.0850$ | $0.2915$ | $0.9217$ | $0.1415$ | $0.0827$ | $0.6147$ |
| **Extreme Gradient Boosting (xgboost)** | $0.1980$ | $0.0895$ | $0.2992$ | $0.9176$ | $0.1325$ | $0.0831$ | $0.0778$ |
| **Gradient Boosting Regressor (gbr)** | $0.2457$ | $0.1197$ | $0.3460$ | $0.8898$ | $0.1649$ | $0.1118$ | $0.2559$ |
| **Decision Tree Regressor (dt)** | $0.2244$ | $0.1411$ | $0.3756$ | $0.8701$ | $0.1695$ | $0.0908$ | $0.0108$ |
| **K Neighbors Regressor (knn)** | $0.2254$ | $0.1464$ | $0.3826$ | $0.8652$ | $0.1784$ | $0.1003$ | $0.0041$ |
| **Proposed Federated Learning Dew (FedAvg-XGB)** | **$0.2983$** | **$0.1691$** | **$0.4112$** | **$0.8443$** | **$0.1652$** | **$0.1384$** | **$0.2240$** |
| **AdaBoost Regressor (ada)** | $0.3843$ | $0.2322$ | $0.4819$ | $0.7862$ | $0.2487$ | $0.1950$ | $0.1101$ |
| **Linear Regression (lr)** | $0.6695$ | $0.7691$ | $0.8770$ | $0.2917$ | $0.4268$ | $0.3993$ | $0.0008$ |
| **Ridge Regression (ridge)** | $0.6695$ | $0.7691$ | $0.8770$ | $0.2917$ | $0.4268$ | $0.3993$ | $0.0007$ |
| **Bayesian Ridge (br)** | $0.6714$ | $0.7717$ | $0.8785$ | $0.2894$ | $0.4277$ | $0.4005$ | $0.0009$ |
| **Elastic Net (en)** | $0.6728$ | $0.7733$ | $0.8794$ | $0.2879$ | $0.4269$ | $0.3994$ | $0.0008$ |
| **Lasso Regression (lasso)** | $0.6737$ | $0.7759$ | $0.8808$ | $0.2855$ | $0.4269$ | $0.3995$ | $0.0007$ |
| **Huber Regressor (huber)** | $0.6412$ | $0.8610$ | $0.9279$ | $0.2071$ | $0.4560$ | $0.4336$ | $0.0141$ |
| **Passive Aggressive Regressor (par)** | $0.7631$ | $0.9159$ | $0.9570$ | $0.1566$ | $0.6258$ | $0.8035$ | $0.0013$ |
| **Orthogonal Matching Pursuit (omp)** | $0.7981$ | $1.0550$ | $1.0271$ | $0.0285$ | $0.4802$ | $0.4813$ | $0.0006$ |
| **Dummy Regressor (dummy)** | $0.8427$ | $1.0862$ | $1.0422$ | $-0.0003$ | $0.4818$ | $0.4920$ | $0.0001$ |

_________________________________________________________________________________

📁 Repository File Tree

├── Crop_recommendation.csv # Raw Kaggle 22-crop agronomic dataset ├── raw_sensor_data.csv # Simulated raw IoT sensor telemetry stream ├── enriched_sensor_data.csv # Preprocessed stream with Quality & Criticality ├── universal_crop_xgboost.json # Compiled edge XGBoost AI classifier ├── crop_label_encoder.pkl # 22-crop categorical label encoder ├── private.pem / public.pem # ECDSA secp256k1 cryptographic keypair ├── dew_local_buffer.jsonl # Offline Dew local buffer (2,200 records) ├── ipfs_sensor_batch_snapshot.json # Canonical IPFS snapshot (CID & Merkle Root) ├── dasp_blockchain_ledger.json # Real decentralized on-chain ledger ├── DASP_DAFO_experiment_log.csv # Microsecond benchmark latency records │ ├── step1_preprocess.py # Telemetry simulation & preprocessing ├── step2_train_models.py # Edge XGBoost model training ├── step3_dasp_dafo_engine.py # Dew node inference & cryptographic chain ├── step4_cloud_sync.py # DAFO queue & cloud signature verification ├── step5_evaluate_metrics.py # Microsecond latency benchmarking ├── step6_plot_graphs.py # Latency & buffer graph generation ├── step7_ipfs_web3_verify.py # IPFS CIDv0 & Merkle Tree generator ├── step8_real_blockchain_miner.py # Real blockchain miner & block settlement ├── step9_plot_heatmaps.py # Unbiased CV, confusion matrix & correlation heatmaps ├── step10_plot_access_control.py # IEEE/ACM registration & access control diagram generator ├── step11_plot_dew_comparisons.py # Quantitative Dew layer benchmarks & comparative diagrams ├── step12_plot_pdr.py # Packet Delivery Ratio (PDR) evaluation generator ├── step13_plot_bandwidth.py # Bandwidth usage, payload overhead & rural cost benchmarks ├── step14_plot_cloud_transactions.py # Cloud transaction efficiency & batch scaling evaluator ├── step15_plot_fairness.py # DAFO multi-objective fairness & Jain index evaluator ├── step16_benchmark_regression_fl.py # Regression & Federated Learning benchmarking script ├── interactive_predictor.py # CLI manual test tool ├── metamask_anchor.html # Web3 MetaMask anchoring interface │ ├── scientific_paper_system_arch.jpg # IEEE/ACM system architecture diagram ├── keypad_phone_sd_workflow.jpg # Keypad phone hardware diagram ├── Smartphone Dew Cache Workflow Diagram.png # Smartphone Dew cache workflow diagram ├── crop_confusion_matrix.png # 22-Crop unbiased confusion matrix ├── correlation_matrix_heatmap.png # Feature correlation heatmap ├── latency_breakdown.png # Component latency breakdown ├── latency_stability.png # Latency jitter stability stream ├── buffer_growth.png # Buffer memory growth curve ├── blockchain_registration_access_control.png # Node access control lifecycle diagram ├── dew_layer_comparison_diagram.png # Dew computing 4-panel comparison diagram ├── packet_delivery_ratio_analysis.png # PDR vs link loss & blackout duration curves ├── bandwidth_usage_analysis.png # Bandwidth consumption & transmission efficiency analysis ├── cloud_transactions_analysis.png # Cloud & blockchain transaction volume comparison ├── fairness_ordering_analysis.png # DAFO fairness score & Jain index evaluation ├── Screenshot 2026-10-14 at 9.10.32 AM.png # MetaMask portal initial connection ├── Screenshot 2026-10-14 at 9.15.02 AM.png # MetaMask EIP-712 signature request ├── Screenshot 2026-10-14 at 9.15.09 AM.png # Confirmed cryptographic on-chain settlement ├── Screenshot 2026-10-14 at 10.45.04 AM.png # Live Field 1 Soil Moisture stream ├── Screenshot 2026-10-14 at 10.45.10 AM.png # Live Field 2 Temperature stream ├── Screenshot 2026-10-14 at 10.45.16 AM.png # Live Field 3 pH stream ├── Screenshot 2026-10-14 at 10.45.23 AM.png # Live Field 4 Nitrogen (N) stream ├── Screenshot 2026-10-14 at 10.45.29 AM.png # Live Field 5 Phosphorus (P) stream └── Screenshot 2026-10-14 at 10.45.36 AM.png # Live Field 6 Potassium (K) stream

_________________________________________________________________________________

📜 Citation & License

This research framework is licensed under the MIT License. If utilizing the DASP-DAFO architecture, algorithms, or benchmark models in academic publications, please cite the framework as:

@article{dasp_dafo_2026, title={DASP-DAFO: Dew Assisted Sensor Protocol with Data Anomaly Fair Ordering and Blockchain Anchoring for Rural IoT Precision Agriculture}, author={DASP-DAFO Research Consortium}, journal={IEEE Internet of Things Journal / ACM Transactions on Sensor Networks}, year={2026} }